Utilities Excavation Waste Assessment

Site Assessment

This assessment is to be completed after opening the excavation and once relevant utility asset is exposed / ready for installation.

| Utility Type: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Date of Assessment: |  |  |  |  |  |  |
| Location of work: | ☐ | Public |  | ☐ | Private |  |
| Permit Number: |  |  |  |  |  |  |
| Work Order Ref: |  |  |  |  |  |  |
| Excavation Site Number |  |  |  |  |  |  |
| Address: |  |  |  |  |  |  |
| Post code |  |  |  |  |  |  |
| Highway Authority: |  |  |  |  |  |  |
| Works Type: | ☐ | Immediate |  | ☐ | Minor |  |
|  | ☐ | Standard |  | ☐ | Major (TM Only) |  |
| Surface Location: | ☐ | Carriageway |  | ☐ | Footway / Footpath |  |
|  | ☐ | Verge |  | ☐ | Other |  |
| Optional – What Three Words |  |  |  |  |  |  |

| Question: |  | Answer: |  |  |  |  |  |  |  | Notes |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Q1 | Are there any signs of asbestos fibres or asbestos containing materials in the excavation? | ☐Yes |  |  | ☐No |  |  |  |  | If asbestos / signs of asbestos are identified the excavation does not qualify for a risk assessment. |  |  |
| Questions – Asphalt / Bitumen Road Surfaces: |  |  |  |  |  |  |  |  |  |  |  |  |
| Q2 | Is the binder shiny, sticky to touch and is there an organic odour? | ☐Yes |  |  | ☐No |  |  |  |  | All three (shiny, sticky and Creosote odour) required for a “yes”. |  |  |
| Q3 | Spray PAK across the profile (depth) of asphalt / bitumen. Does the paint change colour to Band 1 or 2? | ☐Yes |  | ☐No |  | ☐N/A |  |  |  | Ensure to spray a line across the full depth of the bituminous layer. Refer to PAK colour chart. |  |  |
| Questions – All mobilised wastes / materials: |  |  |  |  |  |  |  |  |  |  |  |  |
| Q4 | Is the soil stained an unusual colour (such as an orange, black, blue, green)? | ☐Yes |  |  | ☐No |  |  |  |  | Compare the discolouration of soil to other parts of the excavation. |  |  |
| Q5 | If there is water / moisture in the excavation – is there a rainbow sheen / colouration to the water? | ☐Yes |  |  | ☐No |  |  |  |  | Looking for signs of oil in the excavation. |  |  |
| Q6 | Are there any pungent odours to the material? | ☐Yes |  |  | ☐No |  |  |  |  | Think bleach, garlic, egg, tar, gas, pungent, sweet smells. |  |  |
| Q7 | Use Litmus paper on wet soil, does it change colour to High or Low pH? | ☐Yes |  |  | ☐No |  | ☐N/A |  |  | Refer to pH colour chart. |  |  |
| Assessment result(s): |  |  |  |  |  |  |  |  |  |  |  |  |
| Bituminous |  | ☐ | Green |  |  |  |  | ☐ | Red |  | ☐ | N/A |
| Sub-base |  | ☐ | Green |  |  |  |  | ☐ | Red |  | ☐ | N/A |

| Assessor Name: |  |
| --- | --- |

Utilities Excavation Waste Assessment

Sampling and Results

If samples have been taken for testing, please provide details below.

| Sampling details to be filled in on site: |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sample Number |  |  |  |  |  |  |  |  |  |  |  |  |
| Material Sampled |  | ☐ | Bituminous |  |  |  |  | ☐ |  | Sub-base |  |  |
| Lab results to be filled in by reporting team: |  |  |  |  |  |  |  |  |  |  |  |  |
| Lab Analysis |  | ☐ |  | Green |  |  | ☐ |  | Red |  |  |  |
| Determinant: |  |  |  |  | Present: | Concentration if Red: (mg/kg) |  |  |  |  |  |  |
| Coal Tar (determined by BaP) |  |  |  |  | ☐ |  |  |  |  |  |  |  |
| Total Petroleum Hydrocarbons (C6-C40) |  |  |  |  | ☐ |  |  |  |  |  |  |  |
| Heavy Metal |  |  |  |  | ☐ |  |  |  |  |  |  |  |
| Asbestos |  |  |  |  | ☐ |  |  |  |  |  |  |  |
| Other |  |  |  |  | ☐ |  |  |  |  |  |  |  |

| Sampling details to be filled in on site: |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sample Number |  |  |  |  |  |  |  |  |  |  |  |  |
| Material Sampled |  | ☐ | Bituminous |  |  |  |  | ☐ |  | Sub-base |  |  |
| Lab results to be filled in by reporting team: |  |  |  |  |  |  |  |  |  |  |  |  |
| Lab Analysis |  | ☐ |  | Green |  |  | ☐ |  | Red |  |  |  |
| Determinant: |  |  |  |  | Present: | Concentration if Red: (mg/kg) |  |  |  |  |  |  |
| Coal Tar (determined by BaP) |  |  |  |  | ☐ |  |  |  |  |  |  |  |
| Total Petroleum Hydrocarbons (C6-C40) |  |  |  |  | ☐ |  |  |  |  |  |  |  |
| Heavy Metal |  |  |  |  | ☐ |  |  |  |  |  |  |  |
| Asbestos |  |  |  |  | ☐ |  |  |  |  |  |  |  |
| Other |  |  |  |  | ☐ |  |  |  |  |  |  |  |

| Sampled by Name: |  |
| --- | --- |
| Results recorded by Name: |  |

Comments: